Bering Sea Jelly Cruise (Leg 2) – July 28th – August 4th, 2017

Echos – Ben Binder (Night) Jonathan Witmer (Day)

ARIS – Suzanne (Night) Ben & Jonathan

ZooVis – Hongsheng (Night) Stuart (Day) Suzanne Ben

Nets – Mary Beth (Day)

CTD – Ben Jamie (Night) Brandon (Martech Day) Jonathan

ARIS Housings and Echosounder computer set to Local Time (UTC -9). Echosounder data file names saved as UTC

0830 – 120 khz tested, 38 not connecting because of software issue. Talking to Jeff about fix.

0830 – 120 set to 1.024ms, 250w, 200ms ping interval

0900 – safety meeting, calibrating at 930

0930 – attempted calibration. Ducers facing inboard at dock (not straight up and down) going to have to try something else…

1000 – talked to Jeff Condiotty. No idea why I can’t get 38-12 on gpt through ek80. Likely because its old and need to try er60 and 38 standalone without 120.

1100 – attempted with er60 and standalone with no success. Emailed kevin and jeff, and called Alex DeRobertis

1450 – on station 1 preparing to deploy

1617 - D20170729-T001506 deployed station_1

1730 – Zoo Vis in

1910-ZooVis out- bridal caught under Aris plate, Aris mount skewed (as in it got f’d up)

1915- D20170729-T001506 retrieved zoovis

2025 – Ran through ARIS data. Data was collected and it looks ok, but the range was stuck at .7  - 1.3 meters… Not sure why, charging and regrouping

2330 – On station 3 deployed CTD

July 29, 2017

0000 – Deployed Transducer pole collecting on 120kHZ (D20170729-T080434)

0010 – Nets going in

0018 – nets up

0048 – zoovis in all running well

0222 – zoovis up sonar off (D20170729-T102128)

0225 – zoovis still running ARIS battery dead. Charging for a few and then powering up to download.

0310 – departing to station 5 (3.5hours)

0645 – On station 5 CTD going in

0720 – nets in

0754 -  Deployed Transducer pole collecting on 120kHZ (D20170729-T155400)

0800 – Pulled ARIS bottles, would not connect and no light seen through view port. Disconnected battery jumpers on battery bottle to reset power supply. Once reconnected Ethernet was found. SONAR-2 would not connect. Heard ARIS make a little noise, but couldn’t get remote desktop. Connected to SONAR_1 and found remote desktop immediately. SONAR-2 is misbehaving. Deploying station 5 on SONAR_1, will test Sonar-2 when recovered. Connected SONAR-2 hdd to laptop and recovered data. All appeared well, data had been collected. Not sure what is wrong. Charging bottles in housings now for deployment

0900 – new mount for ARIS being made, Bridle kept hanging up. Mounting on front of ZooVis upper plate

1040 – ZooVis in with ARIS all good

1218 - ZooVis out with ARIS

1220 – echosounder off and up (D20170729-T201821) ARIS data is GOOD but ZooVis went down half way. Stuart is fairly certain the LED light source has burned out. Won’t be able to collect anymore zoovis data if this is the case. Firing up at station 7 and giving it a try.

1230 – begin transit to station 7

1745 – on station 7

1750 – ctd in

1800 – nets in and pole down

1812 – echosounder on (D20170730-T021246)

1845 – zooviz going in

2025 – zoo vis recovered transducers recovered (D20170730-T041300)

2040 – transit to station 9

07/30/2017

0032 – arrive station 9 preparing ctd. Starting up ARIS to test and make sure continues running. Stopped after 30 min last time? Not sure why, a file was corrupt though. Some rror?

0035 – ctd in, pole in

0043 – echosounders on (D20170730-T084556)

0050 – ctd recovered nets prepping

0100 – nets recovered

0112 – zoovis in

0252 – zoovis recovered echo sounders shutdown (D20170730-T104448)

0310 – begin transit to station 11

0645 – on station 11

0650 – ctd and echosounders in

0700 – CTD recovered echosounder start (D20170730-T150140)

0720 – zoovis in with aris

0853- zoovis with ARIS out

0905 - Echosounder out (D20170730-T150140)

1350 – on station 13

1400 - Echosounder in (D20170730-T213837)

1410 - ZooVis with ARIS in

1540 - ZooVis with ARIS out

1545 - Echosounder out (D20170730-T213837)

1900 – ARIS files on station 13 were corrupt and did not record data once it left the deck… something happened with software I think… Disconnected ARIS from frame and tested in 50gal drum. Data was recorded, depth sensor read, and images were captured. ARIS is working fine. Not sure why the data was corrupted last time. Changed the record time to 30 minutes (rather than 10) to prevent splitting file interruption (if that’s the problem), enabled a delay feature to help communicate with remote desktop (don’t know if this will help, don’t think so), and also enabled ‘infinite reconnect on timeout’ feature (it will no longer shutdown if network lost for more than 60 seconds). If the issue was network drop for some reason, this will allow for reconnect and automatically begin recording. We will see how it works on station 15

2125 – arrive station 15

2135 – ctd in echosounder in (D20170731-T053521) ARIS is up and running ok

2143 – nets

2150 – zoovis and ARIS in

2322 – aris zoovis up echosounders up (D20170731-T070120)

2330 – ARIS data good headed to station 12

7/31/2017

0620 – arriving at station 12

0630 – echosounder in ctd in (D20170731-T143152)

0635 – ctd up

0645 – nets in

0650 – nets up

0655 – zoovis and ARIS in

0833 – all up (D20170731-T163343)

0850 – proceeding to station 17

1010 – on station 17

1020 – CTD going in

1025 – echosounder on (D20170731-T182423)

1030 – nets going in

1045 – zoovis in and ARIS down

1201 – shutdown echosounders to test new 38 khx fix

1220 – all up (D20170731-T195934)

Station 18

1400 - CTD in

1415 - CDT up

1420 - echosounders in (D20170731-T223055)

1422 - nets deployed

1615 – echosounder up all up (D20170801-T001339)

Station 19

1805 - ctd deployed

1800 – echosounder deployed (D20170801-T020643)

1811- ctd recovered

1818- nets in

1823- nets out

1835  - ARIS in

2000  - ARIS out

2010 – echosounder up (D20170801-T041035)

Station 20

2155 – ctd in

2203 - echosounder on (D20170801-T060345)

2230 – drop outs noticed due to rough weather (5-8ft)

2350 – zoovis and aris up

2353 – echosounder out (D20170801-T075046)

08/01/2017

0010 – headed to station 22

Station 22

0330 – ctd

0338 – nets

0340 – echosounders on (D20170801-T114014)

0350 – aris and zoovis in

0525 – aris and zoovis up

0528 – echo sounder up (D20170801-T132739)

0545 – headed to station 24

0900-echosounder in (D20170801-T165811)

0920- ARIS in

1055-ARIS out and echosounder out (D20170801-T165811)

1100- heading to station 26

Station 26

1410-ctd in

1423-echosounder in (D20170801-T223517)

1447-ARIS in

1617 ARIS out

1620 – echosounder out (D20170801-T223517)

Station 28

1940 – ctd in

1948 – nets in

1951 – echosounder in (D20170802-T034921)

2002 – ARIS in

2135 – ARIS up

2140 – echosounder out (D20170802-T053615)

2145 – headed to station 30

08/02/2017

Station 30

0058 – ctd in

0103 – echosounder on (D20170802-T090338)

0110 – backed up ARIS and echosounder data through station 28

0125 – nets in

0135 – ARIS in

0320 – ARIS up

0328 – echosounder up (D20170802-T112703)

0335 – headed to station 31

Station 31

0430 - ctd in

0435 – echosounder in (D20170802-T123531)

0445 – nets in

0500 – ARIS in

0635 – ARIS up

0642 – echosounder up (D20170802-T144033)

0650 – headed to station 32

Station 32

0750 – ctd in

0805 – nets in

0820 – echosounder in (D20170802-T161910)

0825 – ARIS in

0955-ARIS out

1005- echosounder out (D20170802-T161910)

1010-Heading to station 33

Station 33

1125-Ctd in

1130-echosounder in (D20170802-T193717)

1140-ctd in

1145- nets in

1157-nets out

1211-ARIS in

1345-ARIS out

1347-echosounder out (D20170802-T193717)

Station2

1555 – ctd in

1600 - nets

1610-Echosounder in

1626- ARIS in (D20170803-T001125)

1750 – ARIS up

1755 – echosounder up (D20170803-T015432)

1810 – ARIS power supply blew up in rogue wave. Shutdown ARIS at time of deployment. Redoing this site

Station 2 redo

1818 – ARIS in

1821 – echosounder back in (D20170803-T021851)

1923 – echosounder up (D20170803-T032328)

1925 – ARIS up

1930 – on to station 1

Station 1 replicate

2148 – ctd in

2150 – echosounder in (D20170803-T054908)

2200 – ARIS in

2350 – ARIS and echosounder up (D20170803-T074511)

2355 – headed home! All echosounder data backed up on KBos BU and Hongsheng XHD

Calibrating in the morning (08/03/2017)